CCNPv7 ROUTE

### Chapter 6 Lab 6-1, Configure NAT Services

### Topology

### Objectives

- Configure dynamic NAT and static NAT on the HQ router.

- Configure dynamic NAT on the Branch router.

- Verify the configuration and operation using show commands.

### Background

The HQ and Branch sites must be configured to support NAT. Specifically, the HQ and Branch routers will be configured to provide inside LAN users with outside public addresses using NAT. The HQ router will also provide static NAT to access the Email server from the outside network.

Note: This lab uses Cisco 1941 routers with Cisco IOS Release 15.2 with IP Base. Depending on the router or switch model and Cisco IOS Software version, the commands available and output produced might vary from what is shown in this lab.

### Required Resources

- 3 routers (Cisco IOS Release 15.2 or comparable)

- Serial and Ethernet cables

### Step 1: Configure loopbacks and assign addresses.

Cable the network as shown in the topology diagram. Erase the startup configuration and reload each router to clear previous configurations. Using the addressing scheme in the diagram, apply the IP addresses to the interfaces on the HQ, ISP, and Branch routers.

You can copy and paste the following configurations into your routers to begin.

Note: Depending on the router model, interfaces might be numbered differently than those listed. You might need to alter the designations accordingly.

HQ (R1)

hostname HQ

!

interface Loopback0

description Headquarters LAN

ip address 10.10.10.1 255.255.255.0

exit

!

interface Loopback1

description Simulates the Email Server

ip address 10.10.20.238 255.255.255.255

exit

!

interface Serial0/0/0

description Connection to ISP

ip address 209.165.200.226 255.255.255.248

clock rate 128000

no shut

exit

!

end

ISP (R2)

hostname ISP

!

interface Loopback0

description Simulating the Internet

ip address 209.165.202.129 255.255.255.240

exit

!

interface Serial0/0/0

description Connection to HQ

ip address 209.165.200.225 255.255.255.248

no shut

exit

!

interface Serial0/0/1

description Connection to Branch

ip address 209.165.200.241 255.255.255.248

clock rate 128000

no shut

exit

!

ip route 209.165.200.232 255.255.255.248 Serial0/0/0

ip route 209.165.200.248 255.255.255.248 Serial0/0/1

!

end

Branch (R3)

hostname Branch

!

interface Loopback0

description Branch LAN

ip address 192.168.1.1 255.255.255.0

exit

!

interface Serial0/0/1

description Connection to ISP

ip address 209.165.200.242 255.255.255.248

no shut

exit

!

end

- Verify your configuration by using the show ip interface brief and the show interfaces description command. The output from the Branch router is shown here as an example.

Branch# show ip interface brief | include up

Serial0/0/1               209.165.200.242 YES manual up                    up

Loopback0                 192.168.1.1     YES manual up                    up

Branch#

Branch# show interfaces description | include up

Se0/0/1                        up             up       Connection to ISP

Lo0                            up             up       Branch LAN

Branch#

- From the Branch router, run the following Tcl script to verify connectivity.

foreach address {

209.165.200.241

209.165.202.129

209.165.200.226

} { ping $address }

Branch# tclsh

Branch(tcl)# foreach address {

+>209.165.200.241

+>209.165.202.129

+>209.165.200.226

+>} { ping $address }

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.241, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 12/13/16 ms

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.202.129, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.226, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

Branch(tcl)#

Why do the pings to the ISPs loopback and HQ router address fail?

### Step 2: Configure default static routes on Branch and HQ.

- On HQ, configure a default static route to ISP.

HQ(config)# ip route 0.0.0.0 0.0.0.0 209.165.200.225

- On the Branch router, configure a default static route to ISP.

You can copy and paste the following configurations into your routers.

Branch(config)# ip route 0.0.0.0 0.0.0.0 209.165.200.241

- From the Branch router, run the following Tcl script to verify connectivity.

foreach address {

209.165.200.241

209.165.202.129

209.165.200.226

+>} { ping $address}

Branch# tclsh

Branch(tcl)# foreach address {

+>209.165.200.241

+>209.165.202.129

+>209.165.200.226

+>} { ping $address }

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.241, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 12/13/16 ms

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.202.129, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 12/13/16 ms

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.226, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 28/28/28 ms

Branch(tcl)#

Are the pings now successful?

- Connectivity from the Branch router to external addresses has been established. But could a Branch LAN user successfully reach those external addresses? To verify, initiate pings sourced from the Branch LAN interface to the ISP interface, the ISPs loopback interface, and the HQ Internet interface.  Run the following Tcl script on the Branch router to verify connectivity.

foreach address {

209.165.200.241

209.165.202.129

209.165.200.226

} { ping $address source 192.168.1.1}

Branch# tclsh

Branch(tcl)# foreach address {

+>209.165.200.241

+>209.165.202.129

+>209.165.200.226

+>} { ping $address source 192.168.1.1}

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.241, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

.....

Success rate is 0 percent (0/5)

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.202.129, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

.....

Success rate is 0 percent (0/5)

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.226, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

.....

Success rate is 0 percent (0/5)

Branch(tcl)#

Note: You can also specify the router interface designator (for example, S0/0/0, Fa0/0, or Lo1) as the source for the extended ping, as follows:

Branch# ping 209.165.200.226 source Lo1

Why are the pings unsuccessful?

The ISP cannot route back to the internal private address of the Branch LAN.

- Step 3: Configure NAT on the HQ router.

The HQ and Branch internal LANs will be translated to global public IP addresses using NAT when exiting the corporate network.

The ISP has allocated the 209.165.200.233 – 209.165.200.238 (209.165.200.232/29) pool of public addresses to the HQ site.

The HQ site also has an email server that must be accessible to mobile users and Branch office users. Therefore, static NAT must also be configured to use a public address to reach the email server.

- On the HQ router, create an extended NAT ACL that matches the 10.10.10.0/24 LAN.

HQ(config)# ip access-list extended HQ-NAT-ACL

HQ(config-ext-nacl)# remark Permit Local LAN to use NAT

HQ(config-ext-nacl)# permit ip 10.10.10.0 0.0.0.255 any

HQ(config-ext-nacl)# exit

HQ(config)#

- The NAT pool must identify addresses 209.165.200.232 /29.

HQ(config)# ip nat pool HQ-NAT-POOL 209.165.200.233 209.165.200.237 prefix-length 29

HQ(config)#

- The NAT pool and the ACL must be bound together.

HQ(config)# ip nat inside source list HQ-NAT-ACL pool HQ-NAT-POOL

HQ(config)#

- The email server with private IP address 10.10.20.238 will be statically assigned the last public IP address from the NAT pool, 209.165.200.238. Interface loopback 0 on HQ simulates this server.

HQ(config)# ip nat inside source static 10.10.20.238 209.165.200.238

HQ(config)#

- The LAN interface must be identified as an inside NAT interface, and the Internet interface must be identified as an outside NAT interface.

HQ(config)# interface Loopback 0

HQ(config-if)# ip nat inside

HQ(config-if)# exit

HQ(config)#

HQ(config)# interface Loopback 1

HQ(config-if)# ip nat inside

HQ(config-if)# exit

HQ(config)#

HQ(config)# interface Serial0/0/0

HQ(config-if)# ip nat outside

HQ(config-if)# exit

HQ(config)#

### Step 4: Configure NAT on the Branch router.

- The ISP has allocated the 209.165.200.249 – 209.165.200.254 (209.165.200.248/29) pool of public addresses to the Branch site.

- On the Branch router, create a standard NAT ACL that identifies the 192.168.1.0/24 LAN.

Branch(config)# ip access-list extended BRANCH-NAT-ACL

Branch(config-ext-nacl)# remark Permit Local LAN to use NAT

Branch(config-ext-nacl)# permit ip 192.168.1.0 0.0.0.255 any

Branch(config-ext-nacl)# exit

Branch(config)#

- The NAT pool must identify addresses 209.165.200.232 /29.

Branch(config)# ip nat pool BRANCH-NAT-POOL 209.165.200.249 209.165.200.254 prefix-length 29

Branch(config)#

- The NAT pool and the ACL must be bound together.

Branch(config)# ip nat inside source list BRANCH-NAT-ACL pool BRANCH-NAT-POOL

Branch(config)#

- The LAN interface must be identified as an inside NAT interface, and the Internet interface must be identified as an outside NAT interface.

Branch(config)# interface Loopback 0

Branch(config-if)# ip nat inside

Branch(config-if)# exit

Branch(config)#

Branch(config)# interface Serial0/0/1

Branch(config-if)# ip nat outside

Branch(config-if)# exit

Branch(config)#

### Step 5: Verify NAT Configuration.

- Verify the NAT configuration by using the show ip nat statistics and show ip nat translations commands.

Branch# show ip nat statistics

Total active translations: 0 (0 static, 0 dynamic; 0 extended)

Peak translations: 0

Outside interfaces:

Serial0/0/1

Inside interfaces:

Loopback0

Hits: 0  Misses: 0

CEF Translated packets: 0, CEF Punted packets: 0

Expired translations: 0

Dynamic mappings:

-- Inside Source

[Id: 1] access-list BRANCH-NAT-ACL pool BRANCH-NAT-POOL refcount 0

pool BRANCH-NAT-POOL: netmask 255.255.255.248

start 209.165.200.249 end 209.165.200.254

type generic, total addresses 6, allocated 0 (0%), misses 0

Total doors: 0

Appl doors: 0

Normal doors: 0

Queued Packets: 0

Branch#

As shown above, the pool has been configured and the interfaces assigned. The output of the show ip nat translations command confirms that there are currently no active NAT translations:

Branch# show ip nat translations

Branch#

- Initiate NAT traffic by pinging from the Branch LAN to the ISP interface, ISP’s loopback, the HQ Internet interface, and this time also include the HQ public email server address. Run the following Tcl script on the Branch router to verify connectivity.

foreach address {

209.165.200.241

209.165.202.129

209.165.200.226

209.165.200.238

} { ping $address source 192.168.1.1}

Branch# tclsh

Branch(tcl)# foreach address {

+>209.165.200.241

+>209.165.202.129

+>209.165.200.226

+>209.165.200.238

+>} { ping $address source 192.168.1.1}

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.241, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 12/13/16 ms

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.202.129, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 12/13/16 ms

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.226, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 28/28/28 ms

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 209.165.200.238, timeout is 2 seconds:

Packet sent with a source address of 192.168.1.1

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 28/28/28 ms

Branch(tcl)#

All pings should be successful. Troubleshoot if necessary.

- Verify that NAT is occurring by using the show ip nat statistics and show ip nat translations commands.

Branch# show ip nat statistics

Total active translations: 5 (0 static, 5 dynamic; 4 extended)

Peak translations: 5, occurred 00:00:13 ago

Outside interfaces:

Serial0/0/1

Inside interfaces:

Loopback0

Hits: 40  Misses: 0

CEF Translated packets: 20, CEF Punted packets: 0

Expired translations: 0

Dynamic mappings:

-- Inside Source

[Id: 1] access-list BRANCH-NAT-ACL pool BRANCH-NAT-POOL refcount 5

pool BRANCH-NAT-POOL: netmask 255.255.255.248

start 209.165.200.249 end 209.165.200.254

type generic, total addresses 6, allocated 1 (16%), misses 0

Total doors: 0

Appl doors: 0

Normal doors: 0

Queued Packets: 0

Branch#

Branch# show ip nat translations

Pro Inside global      Inside local       Outside local      Outside global

icmp 209.165.200.249:31 192.168.1.1:31    209.165.200.241:31 209.165.200.241:31

icmp 209.165.200.249:32 192.168.1.1:32    209.165.202.129:32 209.165.202.129:32

icmp 209.165.200.249:33 192.168.1.1:33    209.165.200.226:33 209.165.200.226:33

icmp 209.165.200.249:34 192.168.1.1:34    209.165.200.238:34 209.165.200.238:34

--- 209.165.200.249    192.168.1.1        ---                ---

Branch#

Notice that translations are occurring. The output lists the details of the NAT translations sourced by the 192.168.1.1 Branch LAN IP address, which was translated to public IP address 209.165.200.249.